FUNCTIONAL DESIGN SPECIFICATION

E-Bike Battery Charger Controller

Project: EBC-01

Document: EBC-01-FDS-001  Rev 1.2

Date: July 2026

Platform: Siemens SIMATIC S7-1516-3 PN/DP | TIA Portal V20

HMI: Siemens MTP1500 Unified Comfort

# 1. Document Control

| Rev | Date | Description |
| --- | --- | --- |
| 1.0 | Jun 2026 | Initial issue |
| 1.1 | Jun 2026 | Added HMI specification |
| 1.2 | Jul 2026 | Updated to reflect as-built configuration |

# 2. Scope

This document describes the control system for a lithium battery charger built on a Siemens S7-1516-3 PN/DP PLC with a Siemens MTP1500 Unified Comfort HMI. The system charges a 13S lithium-ion e-bike battery pack (48V nominal) using a CC/CV algorithm with thermal derating and fault handling. All operator interaction is through the HMI. There are no physical pushbuttons or indicator lamps.

# 3. System Overview

The PLC reads battery voltage, charge current and temperature from analog sensors. It computes a current or voltage setpoint depending on the active charging phase and commands a charger power stage via an analog output. The HMI allows the operator to set target voltage and target current, start and stop charging, trigger an emergency stop and reset faults.

The control is structured as three function blocks called from OB1 in a fixed order each scan.

| Block | Purpose |
| --- | --- |
| OB1 | Scales analog inputs, calls all FBs in order, writes pwmOut |
| FB_ChargeController | State machine, PID loop, SOC estimation, session time tracking |
| FB_ThermalDerating | Computes derated current setpoint based on battery temperature |
| FB_FaultHandler | Detects all faults, latches fault flag, handles fault reset |

# 4. I/O and Tag List

## 4.1 Analog Sensors (read as MW tags for simulation flexibility)

| Tag | Address | Type | Engineering Range | Description |
| --- | --- | --- | --- | --- |
| batVoltageSensor | %MW6 | INT | 0 to 100 V | Battery voltage raw reading |
| chargeCurrSens | %MW2 | INT | 0 to maxCurrent A | Charge current raw reading |
| batTempSens | %MW4 | INT | 0 to 80 C | Battery temperature raw reading |

## 4.2 Analog Output

| Tag | Address | Type | Description |
| --- | --- | --- | --- |
| pwmOut | %MW0 | INT | Command to charger power stage, 0 to 27648 |

## 4.3 HMI Memory Bits

All operator commands come from the HMI and are written to internal memory bits. There are no physical pushbuttons.

| Tag | Address | Type | Description |
| --- | --- | --- | --- |
| startCMD | %M0.0 | BOOL | Set by HMI START button |
| stopCMD | %M0.1 | BOOL | Set by HMI STOP button |
| eStop | %M0.2 | BOOL | Set by HMI E-STOP button |
| faultReset | %M0.3 | BOOL | Set by HMI RESET FAULT button |

## 4.4 HMI Setpoint and Display Tags

| Tag | Address | Type | Description |
| --- | --- | --- | --- |
| targetC | %MD10 | REAL | Operator target charge current in A |
| targetV | %MD14 | REAL | Operator target voltage in V |
| socEST | %MW8 | INT | Estimated state of charge in percent for HMI display |
| runningTime | %MW18 | INT | Session running time in seconds for HMI display |

# 5. Analog Scaling

All three analog inputs are scaled from raw 0 to 27648 to engineering units using the formula:

engineering value = (raw / 27648) x range_max

| Tag | Range Max | Unit |
| --- | --- | --- |
| batVoltageSensor | 100 | V |
| chargeCurrSens | maxCurrent (hardcoded 10.0) | A |
| batTempSens | 80 | C |

The analog output is scaled from a REAL current setpoint back to INT using:

pwmOut = REAL_TO_INT((currentSetpoint / maxCurrent) x 27648)

maxCurrent is hardcoded at 10.0A in OB1. This is the hardware limit of the charger power stage and does not change at runtime.

# 6. State Machine

## 6.1 States

| State | Value | Description |
| --- | --- | --- |
| IDLE | 0 | Waiting for START command |
| CC | 1 | Constant current charging at derated targetC |
| CV | 2 | Constant voltage charging at targetV |
| TRICKLE | 3 | Final top-off at 5 percent of targetC |
| DONE | 4 | Charge complete, output zero, waiting for STOP |
| FAULT | 5 | Fault active, output zero, waiting for reset |

## 6.2 Transitions

| From | To | Condition |
| --- | --- | --- |
| IDLE | CC | startCMD rising edge AND no fault AND state = 0 |
| CC | CV | processedV >= targetV |
| CV | TRICKLE | processedC <= 0.1 A |
| TRICKLE | DONE | processedV >= targetV AND processedC <= 0.05 A |
| DONE | IDLE | stopCMD pressed |
| Any state | FAULT | Fault detected by FB_FaultHandler |
| FAULT | IDLE | faultReset pressed AND all fault conditions cleared |

# 7. PID Control

A custom discrete PID runs every scan in OB1 during states CC, CV and TRICKLE. The same PID code runs in all three states. Only the setpoint and feedback signal change per state.

| State | Setpoint | Feedback |
| --- | --- | --- |
| CC | chargeCurrentSP (derated targetC) | processedC |
| CV | targetV | processedV |
| TRICKLE | 0.05 x targetC | processedC |

PID gains Kp, Ki and Kd are taken as inputs to FB_ChargeController from OB1. The integral term and previous error are static variables inside the FB so they persist between scans. The derivative term uses Kd = 0.0 in practice making this a PI controller. Output is clamped to 0 to 27648 before conversion to INT.

The PID integral is reset to zero on every transition into a new state to prevent windup from a previous state carrying over.

# 8. Thermal Derating

FB_ThermalDerating computes the current setpoint passed to the PID in CC and TRICKLE states. It has three regions based on battery temperature:

| Region | Condition | Output |
| --- | --- | --- |
| Normal | temp < 35 C | chargeCurrentSP = targetC |
| Derate | 35 C to 45 C | chargeCurrentSP = targetC x (1 - (temp - 35) / 10) |
| No output | temp > 45 C | Overtemperature fault raised in FB_FaultHandler |

The derating output is only used during CC and TRICKLE states. During CV the PID controls voltage so the current setpoint is not used.

# 9. Fault Handling

## 9.1 Fault Detection

FB_FaultHandler runs before FB_ChargeController every scan. It checks five conditions and sets the fault flag to TRUE if any are met. The fault flag is latched and can only be cleared when all conditions are simultaneously false and the operator presses faultReset.

| Fault | Detection Condition |
| --- | --- |
| Overvoltage | processedV > targetV x 1.05 |
| Overtemperature | processedTemp > 45.0 C |
| Charge timeout | runningTime > 28800 seconds (8 hours) |
| Overcurrent | processedC > targetC x 1.10 |
| E-Stop | eStop = TRUE (HMI button pressed) |

## 9.2 Fault Reset

The fault flag clears only when all five conditions are simultaneously false AND faultReset is pressed. If any fault condition is still active, faultReset is ignored. When eStop triggers a fault, runningTime is also reset to zero.

## 9.3 Safe State

When a fault is detected, FB_ChargeController forces state to 5 and sets pwmOut to 0. The system stays in FAULT until the operator clears the condition and presses RESET FAULT on the HMI.

# 10. State of Charge Estimation

A simple open circuit voltage based SOC estimate is computed in OB1 and written to the global tag socEST for HMI display. It is informational only and not used in any control logic.

socEST = ((processedV - V_min) / (V_max - V_min)) x 100

For a 13S lithium-ion pack: V_min = 39.0 V (13 x 3.0 V), V_max = targetV. The result is clamped to 0 to 100 percent.

# 11. HMI Specification

## 11.1 Hardware

| Item | Value |
| --- | --- |
| Model | Siemens MTP1500 Unified Comfort |
| Connection | PROFINET to PLC |
| Engineering software | TIA Portal WinCC Unified |

## 11.2 Screen Layout

The HMI has a single main operating screen with the following zones:

| Zone | Contents |
| --- | --- |
| Header | Project title, date and time, connection status indicator |
| Left | Battery icon with SOC bar and percentage, fills proportional to socEST |
| Center upper | Status banner showing current state text and background color per state |
| Center lower | Four tiles showing live Voltage, Current, Temp and Time values |
| Right | IO fields for operator to enter targetV and targetC with range labels |
| Bottom | START, STOP and RESET FAULT buttons |

## 11.3 Status Banner Colors

| State | Background Color |
| --- | --- |
| IDLE (0) | Light gray |
| CC CHARGING (1) | Light blue |
| CV CHARGING (2) | Light blue |
| TRICKLE (3) | Light blue |
| DONE (4) | Light green |
| FAULT (5) | Red |

## 11.4 Operator Controls

| Button | Tag Written | Action |
| --- | --- | --- |
| START | startCMD %M0.0 | SetBitWhileKeyPressed, transitions IDLE to CC |
| STOP | stopCMD %M0.1 | SetBitWhileKeyPressed, transitions DONE to IDLE |
| RESET FAULT | faultReset %M0.3 | SetBitWhileKeyPressed, clears fault when conditions met |
| E-STOP | eStop %M0.2 | SetBitWhileKeyPressed, triggers immediate fault from any state |

## 11.5 HMI Tag Bindings

| HMI Element | PLC Tag | Direction |
| --- | --- | --- |
| Status banner | chargeState (via FB IDB) | Read |
| Voltage tile | processedV (via FB IDB) | Read |
| Current tile | processedC (via FB IDB) | Read |
| Temp tile | processedTemp (via FB IDB) | Read |
| Time tile | runningTime %MW18 | Read |
| SOC bar and percent | socEST %MW8 | Read |
| Target Voltage field | targetV %MD14 | Read/Write |
| Target Current field | targetC %MD10 | Read/Write |
| START button | startCMD %M0.0 | Write |
| STOP button | stopCMD %M0.1 | Write |
| RESET FAULT button | faultReset %M0.3 | Write |
| E-STOP button | eStop %M0.2 | Write |

# 12. Battery Specification

The system is designed for a 13S lithium-ion e-bike battery pack with the following reference parameters:

| Parameter | Value |
| --- | --- |
| Cell configuration | 13S (13 cells in series) |
| Nominal voltage | 48V |
| Full charge voltage | 54.6V (13 x 4.2V) |
| Minimum voltage | 39.0V (13 x 3.0V) |
| Typical charge current | 5 to 10A (0.5C for 10-20Ah pack) |
| Max charge current (hardware limit) | 10.0A |
| Charge temperature range | 0 to 45 C |
| Derate start temperature | 35 C |
| Derate cutoff temperature | 45 C |

# 13. Test Sequence

The following sequence validates correct operation using the watch table and SIM table in PLCSIM or real hardware.

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | Set targetV = 54, targetC = 5. Set batVoltageSensor = 13000, chargeCurrSens = 13824, batTempSens = 8960 | Initial values loaded |
| 2 | Press START on HMI | State goes to 1 (CC), pwmOut starts rising |
| 3 | Increase batVoltageSensor to 14930 | State transitions to 2 (CV) |
| 4 | Decrease chargeCurrSens to 277 | State transitions to 3 (TRICKLE) |
| 5 | Keep voltage at 14930, decrease chargeCurrSens to 138 | State transitions to 4 (DONE) |
| 6 | Press STOP on HMI | State returns to 0 (IDLE) |
| 7 | Set batTempSens to 15552 | State goes to 5 (FAULT) |
| 8 | Clear batTempSens to 8960, press RESET FAULT | State returns to 0 (IDLE) |
| 9 | Press E-STOP during CC state | State goes to 5 (FAULT) immediately |

End of Document